3DCellAtlas User Manual

- Image Acquisition

Acquire confocal z-stacks of the organ of interest. Particular attention should be paid to achieve the highest possible signal-to-noise ratio of cell wall to cell interior for ideal segmentation, and using a thin enough slice interval to attain the best possible resolution along the z-axis.

- Importing images into MorphoGraphX

Figure 1. Main Window of MorphoGraphX. The mouse pointer highlights the path to open a z-stack.

To open a 3D stack in MorphoGraphX (MGX), the image needs to be in .tiff format. The .tiff file can either be dragged onto the open MGX window, or it can be accessed via Stack > Stack 1 > Main > Open (Figure 1).

Figure 2. Options in the Main and View tab required for interaction with clipping planes. (A) The Control-Key Interaction determines which clipping plane is controlled. (B) In the View tab, the orientation of each clipping plane can be visualised by ticking “Grid”, and enabled by ticking “Enabled”

A

B

In the “View” tab, clipping planes 1, 2 and 3 are tools to slice into the 3D image. Clip 1 does so in the y-z-plane, clip 2 in the x-z-plane, and clip 3 in the x-y-plane. Each clipping plane can be visualised on the image by ticking the box for “Grid”, and enabled by ticking “Enable”. To control the movement of the clipping planes, make sure the correct clip is selected in the main tab under “Control-Key Interaction” (Figure 2A). Holding down the Ctrl-key and clicking the left mouse button will rotate the plane, while right-clicking moves the plane, keeping the same orientation. Move the slider below each clip to adjust the thickness of the clipping plane.

- 3D Segmentation of Cells

Figure 3. Close-up of the Process tab. The arrow points to the tool used for blurring as a preparation for segmenting the image. The adjustable parameter at the bottom is the radius of the blurring, in μm.

Prior to the 3D segmentation of cells, the image needs to be blurred. This is done using “ITK Smoothing Recursive Gaussian Blur”, in the “Process” tab, under “Stack” > “ITK” > “Filters” (Figure 3).

The parameter is set to a radius of 0.5 μm.

Figure 4. Close-up of the Process tab. Tools for copying and swapping main and work stack are shown in the drop-down menu “Multi-stack”.

The result of each modification to the main stack will automatically be transferred into the work stack. Stacks can be saved from either position, and can be copied or transferred from one to another via tools in the “Process” tab, under “Stack” > “Multi-stack” (Figure 4).

Figure 5. Close-up of the Process tab. Tools for segmenting a stack using ITK segmentation algorithms and the adjustable parameters for the “ITK Watershed Auto Seeded” segmentation process are shown.

The blurred stack is then segmented, using the “ITK Watershed Auto Seeded” tool, in the “Process” tab, under “Stacks” – “ITK” – “Segmentation” (Figure 5).

From the adjustable parameters typically only the level is changed from its default to a value between 250 and 700. This threshold refers to the signal intensity for the detections of “edges” in the segmentation process. This value will have to be adjusted to suit each image stack (depending on how the image was acquired on the confocal). A threshold that is too high will cause thinner cell walls with low signal intensity to be missed and cells to be merged, while a threshold that is too over-segments cells into numerous segments. It is worth noting that oversegmented cells can be corrected, while merged cells cannot.

- Editing of 3D Segmented Cells

Figure 6. MGX tools used for editing stacks.

The segmented stack now needs to be visually inspected, and cells that have mistakenly been assigned more than one label (“oversegmented”) can be corrected by editing. The tools for this can be found in the tool bar of the MGX window (Figure 6).

The dropper icon is used to select a label from the work stack, by holding down the Alt-key and left mouse click. The number of the label will appear on the bottom left corner of the MGX window.

To replace another label with the selected one, the paint bucket tool is selected. Clicking on the label to be changed while holding down the Alt-key will replace the original label with the active selected one.

To edit cells on the inside of the stack, any of the three clipping planes can be used to move through the image. Progressive changes during editing of the segmented image stack need to be saved frequently, as there is no “undo” function in MGX.

- Creating the 3D Cell Mesh

Once the segmented image is fully edited, a cell mesh describing the surface of each individual cell is created. The tool for this process is called “Marching Cubes 3D”, and can be found in the “Process” tab, under “Mesh” > “Creation” (Error! Reference source not found.).

Under the parameters, “Cube size” determines the distance between each point in the mesh; this will have to be adjusted to suit the size of the cells. A cube size of 2 μm is typically used.

“Min Voxels” determines the minimum size in voxels of a segmented cell to be assigned a surface.

Figure 7. Tools and parameters for creating meshes. (A) Algorithms used to create meshes. (B) Adjustable parameters for the “Marching Cubes 3D” tool. (C) Adjustable parameters to create a surface mesh with the “Marching Cubes Surface” tool.

A

B

C

“Smooth passes” is the number of times the mesh is smoothed during its creation. Repeated smoothing will shrink segmented cell meshes. This procedure must be used sparingly, if not at all.

“Label” refers to the cell ID the mesh is being created on. If a number is chosen, the marching cubes will only run on this particular label; if it is "0" it runs on the whole sample.

- Creating the Surface Mesh

A second mesh which describes the surface of the whole organ to be analysed needs to be created. For this, the original image is blurred with the “ITK Smoothing Recursive Gaussian Blur”, at a radius of 5 μm (see Figure 3), and the surface mesh is created using the “Marching Cubes Surface” tool, in the “Process” tab, under “Mesh” > “Creation” (Error! Reference source not found.). The cube size is typically set to 5 μm for this mesh (Figure 7C).

The threshold refers to the signal intensity. This will have to be adjusted to suit each image to get a smooth description of the surface.

- Trimming the open end of the Surface Mesh

Figure 8. Screenshot of the MGX window showing a surface mesh from which one end has been selected using clip 1.

The end of the surface mesh, where the sample was truncated during imaging needs to be cut off for 3DCellAtlas. Surface meshes of hypocotyls have two “open ends”, while embryo axes and roots where the tip is included, need to be cut at one end only. To cut ends of a surface mesh, use any of the clip planes. Move the clip plane to include the end to be cut off and select it by running “Select Clip Region”, in the “Process” tab, under “Mesh” > “Selection”. The selected region will be highlighted in red (Figure 8). Pressing the Delete button on the keyboard will delete the selected region. The final surface mesh with the end(s) cut off is then saved.

Figure 9. Close-up of the View tab. (A) To create a Bezier plane, “Draw”, “Grid” and “Bezier” need to be ticked before pressing “Reset”. (B) Tools for editing meshes and Beziers. The arrow points to the tool for selecting and moving Bezier handles.

A

B

Creating the Bezier Curve

Figure 10. MGX window with a Bezier line through the centre of a root. The mouse pointer has been dragged over one of the handles while holding down the Alt-key to select it.

To create a Bezier curve that describes the geometric centre of the radial sample, in the “View” tab, under Cutting Surface, tick “Draw”, “Grid”, “Bezier”, then click “Reset” (Figure 9A). This will generate a 2D Bezier plane in the x-y plane with 5x5 handles. To collapse these into a single line, go to Process > Mesh > 3D Cell Atlas > Tools, “Collapse Bezier Points” (see Figure 12). These five handle control points need to be adjusted to fit the Bezier line to the centre. The “Select points in mesh” (Figure 9B, arrow) tool is used to select handles, by holding down the Alt-key and clicking on, or dragging over, a control handle (Figure 10). Selected handles will turn red. These can be moved by simultaneously holding down the Alt- and Shift-key while clicking and dragging them to the required position. The positions of the Bezier points are stored in MGXV files, and need to be saved the top main menu via “File” > “Save as”.

- Preparing the Sample for 3DCellAtlas

To run 3DCellAtlas, open the Bezier view file, load the segmented cell mesh into “Mesh 1”, and the organ surface mesh into “Mesh 2”. This can be saved again as the current MGXV file. In this way opening the MGXV file will open the cell mesh, organ surface mesh and the Bezier curve, all together in the correct locations.

Highlight a cortical cell at one end of the sample in the segmented mesh using the “Select Connected Area”  tool with a left click while holding down the Alt-key. This cortical cell will identify cell position “1” for the cortical referencing system.

If the organ has two different cell topologies, for example roots where the root cap splits, or embryo axes which have both radicle and hypocotyl cellular arrangements, an additional cell on the outside of the sample needs to be highlighted. The position where the cell topology changes, is where this second outside cell should be selected. This is not required for organs with a single cell topology, for example hypocotyls (Figure 11).

Figure 11. Screenshots of MGX windows of the set-up required for running the 3D Cell Atlas tools. (A)  Arabidopsis embryo with two cells highlighted (first cortical cell and a cell at the position of the topological switch). (B) Arabidopsis seedling hypocotyl with the first cortical cell highlighted.

A

B

Figure 12 Close-up of the Process tab. All algorithms and tools from the “Cell Atlas 3D” drop-down menus are shown. The adjustable parameter at the bottom belongs to the “B – Assign Cell Types” function. It refers to segments within the organ to be analysed where cell topologies change.

Step 1 of 3DCellAtlas: A -Analyze Cells 3D

In the “Process” tab, under “Mesh” > “Cell Atlas 3D”, “A – Analyze Cells 3D” will calculate all geometric data of the mesh, and determine the positions of all cells relative to the Bezier (centre) and the surface (Figure 12). In the parameters, a minimum volume to be identified as a cell can be set.

- Step 2 of 3DCellAtlas: B –Assign Cell Types

Figure 13. Screenshots of the GUI opened upon running “Assign Cell Types”. (A) If an organ with two topologies is analysed, these two regions will be separately annotated, and appear as segment 1 and segment 2 in a drop-down menu next to “Part of organ”. (B) This option is disabled for organs with a single topology.

A

B

Next, run “B – Assign Cell Types” (Figure 12). For organs with a single cell topology, the parameter “Has multiple segments” needs to be “No”. If the organ has two different topologies, the parameter needs to be “Yes”. Pressing “GO” will prompt a GUI to open (Figure 13).

The drop-down next to “Part of organ”, with the options “segment 1” and “segment 2” (Figure 13A) refers to the parts of the organ with different topologies, divided at the second selected cell prior to analysis. This option will be disabled if “b – Assign Cell Types” was run with the parameter “No” (Figure 13B). By changing the selection of the segment (1 or 2), the heatmap will show all cells located within each respective region. This facilitates the selection of cell clusters across organs with multiple topologies. Without this feature, cell clusters of diverse cell types would overlap.

- Step 3 of 3DCellAtlas: Selecting cell clusters

The 2D heatmap GUI generates cell clusters by plotting the radial distance of cells from the central Bezier curve on the x-axis, and a chosen geometric property of cells along the y-axis. The x-dimension (“X-Dim”) is always set to be the radial distance between the Bezier and the surface mesh. The y-dimension “Y-Dim” can be changed on the drop-down menu to display circumferential, radial or longitudinal cell length. The best option for ideal spread of cell clusters can be chosen to define cell types accurately. Tick the box next to “Show Cell” to display each cell on the heatmap.

The value of “Sigma” is the variance parameter for the Gaussian. A larger sigma will create larger overlap of the "heat" of single cells which leads to a smoother heatmap.

In this 2D heatmap GUI, left mouse clicks are used to place seeds upon cell clusters, and to assign these cell cluster (representing distinct cell types) unique numbers. It is important to keep track of labels assigned to cell types, and to make sure these cell identity labels match across different segments in samples with two different topologies.

Clicking onto a cluster will place a seed onto it, automatically assigns this the number “1”. To change the cell identity label a seed gives to a cluster, hover the mouse over the seed and press a number key on the keyboard (between 0-9). To remove seeds GUI, click and drag them out of the visible area.

We recommend a cell identity numbering system where outside cells are given number 1 and numbers increase towards the middle with each progressive cell layer.

1	epidermis

2	outer cortex

3	inner cortex

4	endodermis

5	vasculature

6	air spaces

7	lateral root cap

Additional layers including the lateral root cap and columella are assigned higher numbers.

The “Auto Clustering” button automatically identified the selected number of clusters by placing seeds on the greatest peaks present on the heatmap.

By pressing “Preselect cluster” after having placed a seed onto one cell cluster, all cells assigned to this cluster will be given this particular label, and will be removed from the heatmap. This will also re-scale the heatmap with the remaining cells.

Figure 14. Heatmaps and locations where annotated seeds are placed when identify cell clusters in 3DCellAtlas. (A-C) Arabidopsis hypocotyl. (D-I) Arabidopsis root showing segment 1 in (D-F) and segment 2 in (G-I). (J-O) Arabidopsis embryo showing segment 1 in (J-L) and segment 2 in (M-O). Yellow labels indicate true cell clusters and red labels those for segmented air spaces.

Cell type labels for all cells are stored as parent labels, and following GUI selection, cells are false-coloured based on their parent label. If cells were misannotated, it is possible to return to 2D Heatmap GUI in “B – Assign Cell Types” and iteratively adjust seed locations to improve accuracy. Typical distribution of cell clusters and placement of seeds for an Arabidopsis hypocotyl, root and embryo axis are shown in Figure 14.

- Step 4 of 3DCellAtlas: C- Assign columella

Figure 15. Parameters for the “Assign Columella” function.

For roots and embryos, cells in front of the highlighted first cortical cell are not included in the cell type analysis described above. To assign columella and lateral root cap at this end of the sample, run “C - Assign Columella”. In the parameters (Figure 15), the cell identity value needs to be changed to the number assigned to these cell types, and not a number used previously.

- Step 5 of 3DCellAtlas: D- Assign Cortical Cells

Figure 16. Parameters for the “Assign Cortical Cells” function.

Next, run “D - Assign Cortical Cells”. With the first cortical cell selected, this will assign the cortical cell referencing system of cell positions to all cells in the mesh. Make sure the value in the parameters  (Figure 16) contains the correct number for the cortical cell selected.

- Step 6 of 3DCellAtlas: E- Topological check

Mis-annotated cells can be detected and highlighted by running “E – Topological Check”.

In the parameters (Figure 17A), “Work on Selection” allows a subset of selected cells to be analysed; set to “No” to examine all cells in the sample.

“Threshold Volume” refers to the minimum size of cells to be examined, while “Threshold Wall Area” defines the minimum shared cell wall area between neighbouring cells to be considered as a connection.

Make sure the labels for root cap, air space and vasculature match the earlier selection.

The “Error Limit” is the minimum number of wrong connections (impossible edges) between cell types for a given cell to be identified as mislabelled.

Figure 17. Parameters for (A) Topological Check and (B) Examine Vasculature.

A

B

16) Step 7 of 3DCellAtlas: F- Examine Vasculature

Following the topological check, run “F – Examine Vasculature”. Make sure the correct parent labels are assigned to vasculature, air space and endodermis (Figure 17B). This tool will automatically reannotate cells that have been mislabelled.

- Step 8 of 3DCellAtlas: Saving and Loading Data

Once all parent labels have been correctly identified, save all cell data. This is done using “Save Cell Data”, under “Cell Atlas 3D” > “Tools”. This stores geometric data for all cells, which cells have been highlighted, the locations of each seed placed in the 2D Heatmap GUI, cell position according to the referencing system (in the column “associated cortical cell”), and parent labels associated with each original unique cell ID in the mesh. Using “Load Cell Data”, all this information can be reloaded onto a mesh.

Figure 18. Drop-down menu for the “Display Cell Data” tool. Any of the geometric data can be chosen, and will be displayed as false colours on the mesh.

Step 9 of 3DCellAtlas: Displaying Cell Data

The “Display Cell Data” tool can be used to visualise and validate measured cell data (Figure 18). These will be displayed in the form of a false-coloured heatmap on the analysed organ. The dropdown menu allows the selection of the parameter to visualize.

19) Step 10 of 3DCellAtlas: Statistical Analysis of 3D Cell Anisotropy

To analyse differences in 3D cell shape, data saved as described in step 18 in csv files for multiple samples or time points are placed into separate directories for “control” and “treatment” data. As well, create a directory in which the output of these statistical analysis is to be placed.

Figure 19. Parameters for the “3D growth analysis” tool.

In the “Process” tab, under “Mesh” > “Cell Atlas 3D” > Statistics, run “cell growth analysis 3D”.

If any of the values for “Control folder”, “Treatment folder” and “Output folder” are left blank (Figure 19), the programme will prompt the user to guide it to each missing directory.

Under “Output Type”, either “Control” or “Treatment” can be chosen. This refers to the samples that will be used to generate heatmaps with the result of the analysis.

“Sliding Average” is an optional smoothing function which averages the value for each cell based on its neighbours. The window for this averaging needs to be an odd number, and can be set under “Window”.

The output of this analysis is a number of text files for each and every control or treatment sample analysed, depending on which has been chosen under “Output Type”.

Per sample, two text files are created. The first contains the data for the mean ratio (treatment over control) of all geometric measurements for each cell (“[filename].csv”). The second file contains the value of elongation factors (E-factors) (“[filename]_eFactor.csv”). These text files may be imported into MorphoGraphX as heatmaps to false colour their respective segmented cellular organ mesh files.

An additional file named “Data_CI.csv” is also generated as part of the process output. This file contains the ratio of each metric for each cell type, at each position, in addition to their respective confidence intervals at each data point. Plots can be generated to visualise these data by importing the Data_CI.csv file into a spreadsheet program such as Excel. Note that the metric in the output file will be the smoothed value, if smoothing has been enabled, while the confidence intervals are always relative to the mean, which is unsmoothed.

20) Step 11 of 3DCellAtlas: Statistical Analysis of 3D Reporter Abundance

Figure 20. Screenshot of the MGX window, with a sample mesh and reporter z-stack file loaded. The mouse pointer highlights the “Heatmap” function to be used to calculate reporter abundance in each cell.

To statistically analyse the abundance of a reporter within each cell of the organ in question, heatmaps for reporter abundance need to be created for each individual sample. For this, the segmented mesh of a sample is loaded, together with the z-stack of the reporter channel of that same sample in the main stack 1.

In the “Process” tab, under “Mesh” > “Heatmap”, run “Heatmap” (Figure 20). This will open a window in which Heat Map Type and Heat Map Visualisation can be chosen from drop-down menus (Figure 21). For reporter abundance, Volume and Interior signal are chosen to quantify reporter concentration.

Ticking the box next to “Signal average” will calculate the reporter abundance relative to the volume of each cell, so as to reflect reporter concentration for each cell.

Figure 21. Windows displaying options for generating heatmaps. (A) The type of heatmap can be chosen from Area, Volume or Walls. (B) The drop-down menu for heat map visualisation offers several options; for reporter abundance, “Interior signal” is chosen.

Under “Report to Spreadsheet”, type in a name for the output file to be created. This will be a csv file which contains each cell ID of the sample and its respective reporter abundance.

Save cell data and reporter abundance files for all samples to be analysed in a directory. The “3D Reporter Abundance Analysis” process will recognise pairs of files with the same file name, with the reporter abundance file name containing an additional string that can be chosen by the user. In this way, the cell type and position information can be match to the corresponding reporter abundance file.

In the “Process” tab, under “Mesh” > “Cell Atlas 3D” > Statistics, run “Reporter abundance analysis”. This will prompt the user to direct the programme to the required directory.

In the parameters (Figure 22), if the value for “Input folder” is left blank, the programme will prompt the user to guide it to the correct directory.

To visualise data on a mesh that is not part of the samples to be analysed, a separate sample (in the form of its “cell_data.csv” file) can be set as “Output File”. “Merge With File” will then have to be set as “Yes”.

Figure 22. Parameters for the 3D reporter abundance analysis tool.

Raw data can be optionally filtered in the process by setting “Upper Filter Type” and/or “Lower Filter Type” to one of the options. “No Filter” will include all data points, while “Percentage” or “Value” will cut off data points beyond the given limit. The limits for each filter can then be set to a value defined under “Upper Filter Limit” and “Lower Filter Limit”.

If the mesh to be used for visualisation of these data is loaded into MGX, the results will automatically be loaded onto it.

The data output file, labelled “output_CI.csv” will automatically be created in the same directory. It contains the mean of the reporter abundance for each cell type, at each position, in addition to the confidence intervals at each data point. From there, plots can be generated to visualise these data as described in the growth analysis.